Trouble codes

transmission

General information

n Refer to the How to use this manual section for: general test
conditions, terminology, detailed descriptions of wiring faults
and a general trouble shooter for electrical and mechanical
faults.

a Trouble codes are displayed by using an LED connected to
the data link connector.

n The TCM fault memory can also be checked and erased
using diagnostic equipment connected to the data link
connector (DLC).

Accessing

• Ensure ignition switched OFF.

• Bridge data link connector (DLC) terminals GND and TAT
1

• Connect LED test lamp between terminals FAT and B+ 1.

NOTE: Connect LED test lamp positive connection to DLC
terminal B+.

Switch ignition ON.

Count LED flashes. Note trouble codes. Compare with

trouble code table.

Long flashes indicate the 'tens' 2 [A].

Short flashes indicate the 'units' 2 [C].

For example: Trouble code 12 displayed 2.

Switch ignition OFF. Rectify faults as necessary.