Model:       323 (BG) • 323 Estate 1,6/4x4 (BW) • 323 (BA/BJ) • 626/MX-6
626/Estate • Xedos 6/9 • MX-3/MX-5

Engine management/transmission

| Trouble code identification |  |  |
| --- | --- | --- |
| Flash |  |  |
| type | Fault location | Probable cause |
| 2-digit |  |  |
| 01 | Ignition pulse missing | Wiring, tachometer, ignition coil, distributor, ECM, |
|  |  | TCM |
| 02 | Diesel: Engine speed (RPM) sensor - no signal | Wiring, RPM sensor |
| 02 1 | Petrol: Crankshaft position (CKP) sensor/engine speed (RPM) sensor - no signal | Wiring, sensor supply voltage, CKP/RPM sensor |
| 03 | Camshaft position (CMP) sensor - no signal | Wiring, sensor supply voltage, CMP sensor |
| 04 | Diesel: Crankshaft position (CKP) sensor - no signal | Wiring, sensor supply voltage, CKP sensor |
| 04 2 | Petrol: Crankshaft position (CKP) sensor/engine speed (RPM) sensor - no signal | Wiring, sensor supply voltage, CKP/RPM sensor |
| 05 | Knock sensor (KS) | Wiring, KS |
| 06 | Vehicle speed sensor (VSS) | Wiring, speedometer, VSS |
| 08 | Mass air flow (MAP) sensor/volume air flow (VAF) sensor | Wiring, MAF/VAF sensor |
| 09 | Engine coolant temperature (ECT) sensor | Wiring, ECT sensor |
| 10 | Intake air temperature (IAT) sensor | Wiring, IAT sensor |
| 11 | Intake air temperature (IAT) sensor | Wiring, IAT sensor |
| 12 | Throttle position (TP) sensor | Wiring, TP sensor adjustment, TP sensor |
| 14 | Barometric pressure (BARO) sensor | Wiring, BARO sensor, ECM |
| 15 | Except V6: Oxygen sensor (O2S)/heated oxygen sensor | Intake leak, fuel pressure/pump, wiring, O2S/HO2S, |
|  | (HO2S) - no activity | injector(s), ECT sensor, MAF sensor, spark plugs |
|  | V6: Heated oxygen sensor (HO2S), bank 2 - no activity | Intake leak, fuel pressure/pump, wiring, O2S/HO2S, |
|  |  | injector(s), ECT sensor, MAF sensor, spark plugs |
| 16 | Exhaust gas recirculation (EGR) valve position sensor | Wiring, EGR valve position sensor |
| 17 | Except V6: Oxygen sensor (O2S)/heated oxygen sensor | Intake leak, fuel pressure/pump, wiring, O2S/HO2S, |
|  | (HO2S) - incorrect signal | injector(s), ECT sensor, MAF sensor, spark plugs |
|  | V6: Heated oxygen sensor (HO2S), bank 2 - incorrect | Intake leak, fuel pressure/pump, wiring, O2S/HO2S, |
|  | signal | injector(s), ECT sensor, MAF sensor, spark plugs |
| 18 | Fuel quantity adjuster | Wiring, fuel quantity adjuster/position sensor |
| 19 | Fuel quantity adjuster position sensor | Wiring, fuel quantity adjuster position sensor |
| 22 | Fuel shut-off solenoid | Wiring, fuel shut-off solenoid |
| 23 | Diesel: Fuel temperature sensor | Wiring, fuel temperature sensor |
|  | Petrol: Heated oxygen sensor (HO2S), bank 1 - no | Intake leak, wiring, O2S/HO2S, injector(s), ECT |
|  | activity | sensor, MAF sensor, spark plugs |
| 24 | Heated oxygen sensor (HO2S), bank 1 - incorrect signal | Intake leak, fuel pressure/pump, wiring, O2S/HO2S, |
|  |  | injector(s), ECT sensor, MAF sensor, spark plugs |
| 25 | Fuel pressure regulator control solenoid | Wiring, fuel pressure regulator control solenoid |
| 26 | Evaporative emission (EVAP) canister purge valve | Wiring, EVAP canister purge valve |
| 28 | Exhaust gas recirculation (EC3R) solenoid -vacuum i | Wiring, EGR solenoid |
| 29 | Exhaust gas recirculation (EGR) solenoid - vent | Wiring, EGR solenoid |
| 34 | Idle air control (IAC) valve | Wiring, IAC valve |
| 35 | Fuel pressure regulator control solenoid 2 | Wiring, fuel pressure regulator control solenoid |
| 36 | Glow plug relay | Wiring, glow plug relay |
| 41 | Intake manifold air control solenoid 1 | Wiring, intake manifold air control solenoid |
| 43 | Fuel injection timing sensor | Wiring, fuel injection timing sensor/solenoid |