MAZDA

Model:	323 (BG) • 323 Estate 1,6/4x4 (BW) • 323 (BA/BJ)
626/MX-6 • 626/Estate • Xedos 6/9 • MX-3/MX-5

Year:    1989-00

Engina code:       BP, BP-DOHC, B3, B3E, B6, B6-SOHC, B6-DOHC, B6E,FP, FS, KF, KJ, KL K8, RF, RF-CX, RF-Turbo, ZL, Z5

System:	Mazda EGI • Mazda EDC

Engine management/transmission

Data link connector (DLC) locations

323 - engine bay, LH

626 -1997/MX-6, Xedos 6/9, MX-3 -

engine bay, LH

626/Estate 1998 -» - engine bay, LH

MX-5 - near brake master cylinder

Trouble codes

General information
Except Xedos 9 2,3

n Refer to the How to use this manual section for: general test

conditions, terminology, detailed descriptions of wiring faults

and a general trouble shooter for electrical and mechanical

faults.
n Trouble codes are displayed by using an LED connected to

the data link connector (DLC).
d Except 323 1998-», 626/Estate/MX-5 1997->: Engine

control module (ECM) displays all available diagnostic

information for immobilizer system.
n DLC without wiring/terminals in positions FAT & TAT:

Engine control module (ECM) incorporates transmission

control function.
n DLC with wiring/terminals in positions FAT & TAT:

Transmission control module (TCM) mounted separately.

n The ECM fault memory can also be checked and erased
using diagnostic equipment connected to the data link
connector (DLC).

Xedos 9 2,3

n Refer to the How to use this manual section for: general test

conditions, terminology, detailed descriptions of wiring faults

and a general trouble shooter for electrical and mechanical

faults.
a Trouble codes are displayed by using an LED connected to

the data link connector.
n Engine control module (ECM) displays all available

diagnostic information for immobilizer system.
Q Transmission control module (TCM) mounted separately.
n Data bus connecting ECM to TCM allows faults relating to

both systems to be displayed when accessing ECM fault

memory.
n The ECM/TCM fault memory can also be checked and

erased using diagnostic equipment connected to the data

link connector (DLC).