Model:      323 (BG) • 323 Estate 1,6/4x4 (BW) • 323 (BA/BJ) • 626/MX-6
626/Estate • Xedos 6/9 • MX-3/MX-5

Engine management/transmission

| Ptype 4-digit | Fault location | Probable cause |
| --- | --- | --- |
| P1117 | Engine coolant temperature (ECT) sensor - circuit | Wiring, ECT sensor |
|  | intermittent |  |
| P1120 | Throttle position (TP) sensor - low input | Wiring short to earth, TP sensor |
| P1125 | Throtte position (TP) sensor - Circuit intermittent | Wiring, TP sensor |
| P1130 | Fuel trim (FT) - control limit exceeded | Intake/exhaust leak, fuel pressure/pump, EVAP/EGR |
|  |  | system, wiring, HO2S |
| P1131 | Heated oxygen sensor (HO2S) - low voltage | Intake/exhaust leak, fuel pressure low, hose |
|  |  | connection, HO2S/wiring short to earth, injector(s), |
|  |  | MAF sensor/wiring |
| P1132 | Heated oxygen sensor (HO2S) -high voltage | Intake/exhaust blockage, fuel pressure high, EVAP/ |
|  |  | EGR system, HO2S/wiring short to positive, |
|  |  | injector(s), MAF sensor |
| P1170 | Oxygen sensor (O2S)/heated oxygen sensor (HO2S), | Intake leak, fuel pressure/pump, wiring, O2S/HO2S, |
|  | bank 1 - incorrect signal | injector(s), ECT sensor, MAF sensor, spark plugs |
| P1173 | Oxygen sensor (O2S)/heated oxygen sensor (HO2S), | Intake leak, fuel pressure/pump, wiring, O2S/HO2S, |
|  | bank 2 - incorrect signal | injector(s), ECT sensor, MAF sensor, spark plugs |
| P1182 | Fuel shut-off solenoid | Wiring, fuel shut-off solenoid |
| P1189 | Fuel injection pump position/speed sensor | Wiring, fuel injection pump speed sensor |
| P1190 | Module coding plug | Wiring, module coding plug |
| P1195 | Barometric pressure (BARO) sensor | Wiring, BARO sensor, ECM |
| P1196 | Ignition switch, start signal - circuit malfunction | Wiring, starter motor |
| P1221 | Traction control system (TCS) - malfunction | Wiring, TCS malfunction |
| P1226 | Fuel quantity adjuster position sensor - circuit malfunction | Wiring, fuel quantity adjuster position sensor |
| P1250 | Fuel pressure regulator control solenoid | Wiring, fuel pressure regulator control solenoid |
| P1252 | Fuel pressure regulator control solenoid 2 | Wiring, fuel pressure regulator control solenoid |
| P1279 | fuel quantity adjuster position sensor - range/ performance problem | Wiring, fuel quantity adjuster position sensor |
| P1298 | Fuel quantity adjuster control module - circuit | Wiring, Fuel quantity adjuster control module |
|  | malfunction |  |
| P1312 | Fuel injection time valve | Wiring, fuel injection timing valve |
| P1318 | Fuel injection timing valve position sensor - circuit malfunction | Wiring, fuel injection timing valve position sensor |
| P1319 | Fuel Injection timing valve position sensor - range/ performance problem | Wiring, fuel injection timing valve position sensor |
| P1345 | Camshaft position (CMP) sensor - no signal | Wiring, sensor supply voltage, CMP sensor |
| P1402 | Exhaust gas recirculation (EGR) valve position sensor | Wiring, EGR valve position sensor |
| P1409 | Exhaust gas recirculation (EGR) valve - circuit | Wiring, EGR valve |
|  | malfunction |  |
| P1474 | Engine coolant blower motor relay - Circuit malfunction | Wiring, engine coolant blower motor relay |
| P1485 | Exhaust gas recirculation (EGR) solenoid - vacuum | Wiring, EGR solenoid |
| P1486 | Exhaust gas recirculation (EGR) solenoid - vent | Wiring, EGR solenoid |
| P1487 | Manifold absolute pressure (MAP) sensor solenoid | Wiring, MAP sensor solenoid |
| P1496 | Exhaust gas recirculation (EGR) valve, motor coil 1 | Wiring, EGR valve |
| P1497 | Exhaust gas recirculation (EGR) valve, motor coil 2 | Wiring, EGR valve |
| P1498 | Exhaust gas recirculation (EGR) valve, motor coil 3 | Wiring, EGR valve |