Trouble codes

Immobilizer

General information

323 ->1997, 626/MX-5 -»1996

n No Self-diagnosis information available from immobilizer

control module.
n Access engine control module (ECM) fault memory for

immobilizer related trouble codes.

323 1998-*, 626/MX-5 1997 -*

n Refer to the How to use this manual section for: general test
conditions, terminology, detailed descriptions of wiring faults
and a general trouble shooter for electrical and mechanical
faults.

a Trouble codes are displayed by the immobilizer warning
lamp.

n System malfunction: During first three engine starts, engine
runs for several seconds and then cuts out.

a After third attempt, engine will no longer start.

Accessing

323 1998 -s MX-5 1997 -»

• Turn ignition switch to START position for 2 seconds, then
back to ON.
Wait 2 minutes.

Count warning lamp flashes. Note trouble codes. Compare
with trouble code table.
Long flashes indicate the LH digit 1 [A].
Short flashes indicate the RH digit 1 [B].
For example: Trouble code 21 displayed 1.
Switch ignition OFF. Rectify faults as necessary.

626 1997 ->

• Trouble codes are displayed by the immobilizer warning
lamp.

Switch ignition ON.

Count warning lamp flashes. Note trouble codes. Compare
with trouble code table.
Long flashes indicate the LH digit 1 [A].
Short flashes indicate the RH digit 1 [B].
For example: Trouble code 21 displayed 1.
Switch ignition OFF. Rectify faults as necessary.

Erasing

323 1998 -s 626/MX-5 1997 -»

• Rectifying faults erases trouble codes.