323/Estate 1,6 8V/16V (BG/BW) • 323 1,8 16V (BG)
626/MX-6 2,5 V6 (GE) • Xedos 6 • Xedos 9 • MX-3 1,6

transmission

Erasing

• Ensure ignition switched OFF.

• Disconnect battery earth lead for at least 20 seconds.

• Depress brake pedal.

• Reconnect battery earth lead.

• Repeat checking procedure to ensure no data remains in
ECM fault memory.

WARNING: Disconnecting the battery may erase memory from
electronic units (e.g. radio, clock).

| Trouble c< | Dde identification |  |
| --- | --- | --- |
| Flash type | Fault location | Probable cause |
| 01 | Ignition pulse missing | Wiring, tachometer, ignition coil, distributor, ECM, |
|  |  | TCM |
| 06 | Vehicle speed sensor (VSS) | Wiring, speedometer, VSS |
| 12 | Throttle position (TP) sensor | Wiring, TP sensor adjustment, TP sensor |
| 14 | Barometric pressure (BARO) sensor | Wiring, BARO sensor, ECM |
| 55 | Input shaft speed (ISS) sensor/turbine shaft speed (TSS) sensor | Wiring, ISS/TSS sensor |
| 56 | Transmission fluid temperature (TFT) sensor | Wiring, TFT sensor |
| 57 | Engine control module (ECM) - gear shift, torque reduction signal 1 | Wiring, ECM, TCM |
| 58 | Engine control module (ECM) - gear shift, torque reduction signal 2 | Wiring, ECM, TCM |
| 59 | Engine control module (ECM) - ECT high, torque reduction signal | Wiring, ECM, TCM |
| 60 | Shift solenoid (SS) A, 1-2 | Wiring, SS |
| 61 | Shift solenoid (SS) B, 2-3 | Wiring, SS |
| 62 | Shift solenoid (SS) C, 3-4 | Wiring, SS |
| 63 | Torque converter clutch (TCC) control solenoid | Wiring, TCC control solenoid |
| 64 | Shift timing solenoid, 3-2 | Wiring, shift timing solenoid |
| 65 | Torque converter clutch (TCC) solenoid | Wiring, TCC solenoid |
| 66 | Transmission fluid pressure (TFP) solenoid | Wiring, TFP solenoid |
| 71 | Closed throttle position (CTP) switch | Wiring, CTP switch |