Model:		626/Estate 1,8/2,0 (GF/GW)

Year:		1997-99

Engine code:	FP, FS

System:		Mazda EGI

Engine management/transmission

Trouble codes

NOTE: Engine control module (ECM) incorporates transmission
control function.

General information

Q Refer to the How to use this manual section for: general test
conditions, terminology, detailed descriptions of wiring faults
and a general trouble shooter for electrical and mechanical
faults.

n Data link connector (DLC) without wiring/terminals in
positions FEN & TEN.

Accessing and erasing

• The engine control module (ECM) fault memory can only be
accessed and erased using diagnostic equipment
connected to the data link connector (DLC).

| Trouble co | de identification |  |
| --- | --- | --- |
| P type | Fault location | Probable cause |
| PO | Refer to EOBD trouble code table | - |
| P1100 | Mass air flow {MAP) censor - circuit Intermittent | Wiring, MAP sensor |
| P1101 | Mass air flow (MAP) sensor - circuit intermittent | Wiring, MAF sensor |
| P1110 | Intake air temperature (IAT) sensor - open/short circuit | Wiring, IAT sensor |
| P1112 | Intake air temperature (IAT) sensor -circuit intermittent | Wiring, IAT sensor |
| P1117 | Engine coolant temperature (ECT) sensor - circuit intermittent | Wiring, ECT sensor |
| P1120 | Throttle position (TP) sensor - low input | Wiring short to earth, TP sensor |
| P1125 | Throttle position (TP) sensor - circuit intermittent | Wiring, TP sensor |
| P1130 | Fuel trim (FT) - control limit exceeded | Intake/exhaust leak, fuel pressure/pump, EVAP/EGR |
|  |  | system, wiring, HO2S |