Model:       323 (BG) • 323 Estate 1,6/4x4 (BW) • 323 (BA/BJ) • 626/MX-6
626/Estate • Xedos 6/9 • MX-3/MX-5

Engine management/transmission

| Flash |  |  |
| --- | --- | --- |
| type | Fault location | Probable cause |
| 2-digit |  |  |
| 46 | Petrol: Intake manifold air control solenoid 2 | Wiring, intake manifold air control solenoid |
|  | Diesel: Closed throttle position (CTP) switch | Wiring, CTP switch |
| 49 | Engine control module (ECM) - defective | ECM |
| 52 | Module coding plug | Wiring, module coding plug |
| 55 | Input shaft speed (ISS) sensor/turbine shaft speed (TSS) sensor | Wiring, ISS/TSS sensor |
| 56 | 323: Engine control module (ECM) - supply voltage | Wiring, ECM |
|  | 626: Transmission fluid temperature (TFT) sensor | Wiring, TFT sensor |
| 57 | Engine control relay | Wiring, engine control relay |
| 60 | Shift solenoid (SS) A, 1-2 | Wiring, SS |
| 61 | Shift solenoid (SS) B, 2-3 | Wiring, SS |
| 62 | Shift solenoid (SS) C, 3-4 | Wiring, SS |
| 63 | Torque converter clutch (TCC) control solenoid | Wiring, TCC control solenoid |
| 64 | Shift timing solenoid, 3-2 | Wiring, shift timing solenoid |
| 65 | Torque converter clutch (TCC) solenoid | Wiring, TCC solenoid |
| 66 | Transmission fluid pressure (TFP) solenoid | Wiring, TFP solenoid |
| 67 | Engine coolant blower motor relay - 1 /low temperature | Wiring, engine coolant blower motor relay |
| 68 | Engine coolant blower motor relay - high temperature | Wiring, engine coolant blower motor relay |
| 69 | Engine coolant blower motor temperature sensor | Wiring, engine coolant blower motor temperature |
|  |  | sensor |
| 71 | Engine control module (ECM)/immobilizer control module - communication error | Wiring, immobilizer control module, ECM |
| 72 | Ignition key - not programmed into ECM | ECM incorrectly/not programmed |
| 73 | Engine control module (ECM)/immobilizer control | Incorrectly programmed immobilizer control module/ |
|  | module - immobilizer codes do not match | ECM |
| 74 | Engine control module (ECM)/immobilizer control module - ignition key codes not match | Incorrect/damaged key, ECM incorrectly programmed |
| 75 | Engine control module (ECM) - EEPROM error | ECM |
| 76 | Engine control module (ECM) — immobilizer code not stored | ECM incorrectly/not programmed |
| 77 | Immobilizer control module - communication error | Incorrect/damaged key, wiring, reader coil, |
|  |  | immobilizer control unit |

1 626/Xedos 6/9, engine codes KF/KL: Located in cylinder block.
2 626/Xedos 6/9, engine codes KF/KL: Located in distributor.

| Ptype 4-digit | Fault location | Probable cause |
| --- | --- | --- |
| PO | Refer to EOBD trouble code table | - |
| P1100 | Mass air flow (MAP) sensor ~ circuit inferfrtifterrt | Wiring, MAF sensor |
| P1101 | Mass air flow (MAP) sensor - eirputt intenpittent | Wiring, MAF sensor |
| P1110 | Intake air temperature (IAT) sensor 2 | Wiring, IAT sensor |
| P1112 | Intake air temperature (IAT) sensor-circuit intermittent | Wiring, IAT sensor |
| P1113 | Intake air temperature (IAT) sensor 2 | Wiring, IAT sensor |