Model:      626/Estate 1,8/2,0 (GF/GW)

Engine management/transmission

| Ptype | Fault location | Probable cause |
| --- | --- | --- |
| P1131 | Heated oxygen sensor (HO2S) - low voltage | Intake/exhaust leak, fuel pressure low, hose |
|  |  | connection, HO2S/wiring short to earth, injector(s), |
|  |  | MAF sensor/wiring |
| P1132 | Heated oxygen sensor (HO2S) - high voltage | Intake/exhaust blockage, fuel pressure high, EVAP/ |
|  |  | EGR system, HO2S/wiring short to positive, |
|  |  | injector(s), MAF sensor |
| P1182 | Fuel shut-off solenoid | Wiring, fuel shut-off solenoid |
| P1189 | Fuel injection pump position/speed sensor | Wiring, fuel injection pump speed sensor |
| P1196 | Ignition switch, start signal - circuit malfunction | Wiring, starter motor |
| P1221 | Traction control system (TCS) - malfunction | Wiring, TCS malfunction |
| P1250 | Fuel pressure regulator control solenoid | Wiring, fuel pressure regulator control solenoid |
| P1298 | Fuel quantity adjuster control module - circuit malfunction | Wiring, fuel quantity adjuster control module |
| P1402 | Exhaust gas recirculation (EGR) valve position sensor - circuit malfunction | Wiring, EGR valve position sensor |
| P1409 | Exhaust gas recirculation (EGR) valve - circuit | Wiring, EGR valve |
|  | malfunction |  |
| P1474 | Engine coolant blower motor relay - circuit malfunction | Wiring, engine coolant blower motor relay |
| P1500 | Vehicle speed sensor (VSS) - Intermittent signal | Wiring, VSS |
| P1504 | Idle air control (IAC) valve - circuit intermittent | Wiring, IAC valve |
| P1602 | Engine control module (ECM)/immobilizer control | Wiring, read coil, ignition key, immobilizer control |
|  | module — communication error | module, ECM |
| P1603 | Ignition key - not programmed Into ECM | ECM incorrectly/not programmed |
| P1604 | Engine control module (ECM) - Immobilizer code not stored | ECM incorrectly/not programmed |
| P1621 | Engine control module PCM)/immobilizer control | Incorrectly programmed immobilizer control module/ |
|  | module - immobilizer codes do not match | ECM |
| P1622 | Engine control module (ECM)/immobilizer control module - ignition key codes not match | Incorrect/damaged key, ECM incorrectly programmed |
| P1623 | Engine control module (ECM) - EEPROM error | ECM |
| P1624 | Engine control module - immobilizer deactivation signal | Battery disconnected - erase trouble code, |
|  | missing | immobilizer system malfunction |
| P1631 | Alternator - output voltage tow | Wiring, alternator drive belt, alternator |
| P1632 | Engine control module - supply voltage tow | Wiring, alternator, battery |
| P1633 | Engine control module - supply voltage high | Wiring, voltage surge when battery disconnected, |
|  |  | alternator |
| P1634 | Alternator - output voltage high/battery voltage low | Wiring, alternator, battery |
| P1649 | Module coding plug | Wiring, module coding plug |
| P1650 | Power steering pressure (PSP) switch | Wiring, PSP switch |
| P1744 | Torque converter clutch (TCC) solenoid | Wiring, TCC solenoid |
| P1765 | Shift timing solenoid, 3-2 | Wiring, shift timing solenoid |