Model:       323 (BG) • 323 Estate 1,6/4x4 (BW) • 323 (BA/BJ) • 626/MX-6
626/Estate • Xedos 6/9 • MX-3/MX-5

Engine management/transmission

| P type 4-digit | Fault location | Probable cause |
| --- | --- | --- |
| P1499 | Exhaust gas recirculation (EGR) valve, motor coil 4 | Wiring, EGR valve |
| P1500 | Vehicle speed sensor (VSS) - intermittent signal | Wiring, VSS |
| P1504 | Idle air control (IAC) valve | Wiring, IAC valve |
| P1508 | Bypass solenoid 1 | Wiring, bypass solenoid |
| P1509 | Bypass solenoid 2 | Wiring, bypass solenoid |
| P1521 | Intake manifold air control solenoid 1 | Wiring, intake manifold air control solenoid |
| P1522 | Intake manifold air control solenoid 2 | Wiring, intake manifold air control solenoid |
| P1523 | Intake manifold air control solenoid 1 | Wiring, intake manifold air control solenoid |
| P1524 | Intercooler bypass solenoid | Wiring, intercooler bypass solenoid |
| P1525 | Air bypass valve solenoid - vacuum | Wiring, air bypass valve solenoid |
| P1526 | Air bypass valve solenoid - vent | Wiring, air bypass valve solenoid |
| P1540 | Air bypass valve control | Hose leak/blockage, wiring, air bypass valve actuator/ |
|  |  | solenoid |
| P1562 | Engine control module (ECM) - supply voltage low | Battery, alternator, wiring |
| P1601 | Data bus, TCM | Wiring, ECM, TCM |
| P1602 | Engine control module (ECM)/immobilizer control | Wiring, read coil, ignition key, immobilizer control |
|  | module — communication error | module, ECM |
| P1603 | Ignition key - not programmed into ECM | ECM incorrectly/not programmed |
| P1604 | Engine control module (ECM) - immobilizer code not stored | ECM incorrectly/not programmed |
| P1606 | Engine control relay | Wiring, engine control relay |
| P1608 | Engine control module (ECM) | ECM |
| P1609 | Engine control module (ECM) — knock control | ECM |
| P1621 | Engine control module (ECM)/immobilizer control | Incorrectly programmed immobilizer control module/ |
|  | module - immobilizer codes do not match | ECM |
| P1622 | Engine control module (ECM)/immobilizer control module - ignition key codes not match | Incorrect/damaged key, ECM incorrectly programmed |
| P1623 | Engine control module (ECM) - EEPROM error | ECM |
| P1624 | Engine control module - Immobilizer deactivation signal | Battery disconnected - erase trouble code, |
|  | missing | immobilizer system malfunction |
| P1627 | ECM/ABS electrical connection - communication error | Wiring, ABS control module, ECM |
| P1631 | Alternator - output voltage low | Wiring, alternator drive belt, alternator |
| P1632 | Engine control module - supply voltage low | Wiring, alternator, battery |
| P1633 | Engine control module -supply voltage low | Wiring, voltage surge when battery disconnected, |
|  |  | alternator |
| P1634 | Alternator - output voltage high/battery voltage low | Wiring, alternator, battery |
| P1649 | Engine control module(ECM) - defective | ECM |
| P1650 | Power steering pressure (PSP) switch | Wiring, PSP switch |
| P1720 | Vehicle speed sensor (VSS) | Wiring, speedometer, VSS |
| P1743 | Torque converter clutch (TCC) control solenoid | Wiring, TCC control solenoid |
| P1744 | Torque converter clutch (TCC) solenoid | Wiring, TCC solenoid |
| P1765 | Shift timing solenoid, 3-2 | Wiring, shift timing solenoid |
| P1770 | Overrunning clutch solenoid | Wiring, overrunning clutch solenoid |
| P1790 | Throttle position (TP) sensor | Wiring, TP sensor |